analysis_pch and analysis_pch_2D plugins for ImageJ

Jay Unruh

Research Advisor

Stowers Institute for Medical Research

Note: please read the License Information file for details about licensing and use.  By using this software, you are agreeing to the enclosed license agreement.

Theory:

The pch analysis plugins for ImageJ were created to facilitate the analysis of confocor 3 raw data files with the photon counting histogram (PCH) algorithm (Mueller 1999).  The fitting algorithm is simple Newton’s method non-linear least squares.  The basic idea behind pch is that multiple molecules flowing in and out of a femtoliter confocal volume (point spread function or PSF) will create an intensity distribution corresponding to their distribution of molecular brightnesses.  Unfortunately, there is an intrinsic distribution of intensities due to the fluctuations in particle number, photon counting shot noise, and spatial variation in the PSF.  In 1999, Mueller, and simultaneously, Kask, wrote down the equations governing these fluctuations and arrived at the equations needed for PCH or FIDA, respectively.  Given the complexity of the FIDA method, I have chosen to write my analysis program based on the PCH method.  Note that some of the units used in the program (brightness and intensity) are relative to the sampling frequency (counts per molecule per sampling interval or counts per sampling interval).  In order to convert to counts per second, simply multiply by the sampling frequency.

2D PCH analysis was subsequently derived by Kask (2000) and Mueller (2005) to analyze bivariate distributions of intensity, allowing for the measurement of hetero-binding interactions and stoichiometries.  The 2D distribution is easily derived from the 1D distribution allowing relatively easy extension of a fitting algorithm to 2D.

There are several significant limitations to the PCH method and other methods like it.  Chief among these is the similarity of the distribution to the poisson distribution that governs the photon counting noise.  As a result, without significant statistics that are typically not available with fluorescent proteins it is only possible to find the average brightness and number of molecules in a sample.  Inclusion of a background component or a second species is simply not justified by the statistics.  Therefore, it is wise to include more information in the fit.  For example, one could fix the molecular brightness of one species or fix the background intensity from a separate measurement.

Global analysis is simply the act of fitting multiple curves simultaneously with least squares methods.  In doing so, one can improve the statistics of a parameter that is shared between the analyses of individual curves.  For example, one might make many measurements of the PCH of transformed mEGFP in yeast cells.  Due to the intrinsic heterogeneity in the yeast protein expression system, one might expect the concentration of mEGFP to vary significantly from sample to sample, but the molecular brightness of mEGFP should remain constant.  Therefore it is possible to link the molecular brightness across all samples and allow the individual number parameters to vary.  For more information about Global analysis see Beechem 1992.

Number and Brightness (N&B) analysis is an extension of a theory developed by Quian and Elson (1990) and later extended by Mueller (2004).  The formalism is described by Digman et al (2008).  This formalism allows for the calculation of average brightness and number using moments (and more completely, cumulants) of the photon count distribution.  The average molecular brightness is given as follows:

ε=σ2−IγI=ifiεi, where i is the molecular brightness of species i (in counts per sampling interval per molecule), fi is the fractional intensity of species i, 2 is the variance, and  is the gamma factor defined elsewhere (Thompson 1991).  For confocal measurements, the gamma factor is 0.3536 and for two photon experiments it is 0.078.  The average number of molecules has a more complex definition, but is simply given by the average intensity divided by the average molecular brightness.

In 2006, Mueller’s lab developed dual color cumulant analysis which was adapted to a dual color brightness formalism by Digman et al in 2009.  The dual color molecular brightness is defined as follows:

εgr=σgr2γIgIr=ifi,gfi,rεi,gεi,r, where the subscripts g and r refer to the green and the red detection channels, and σgr2is the covariance between the green and red signals.  In the absence of bleedthrough, the dual color brightness is zero.  In the presence of bleedthrough, the dual color brightness due to bleedthrough is given as follows:

εgr,bleedthrough=εgβIgIr, where  is the ratio between the red channel intensity of the green species and the green channel intensity of the green species.

Experimental Guidelines:

The important factor to remember in doing PCH experiments is that all fluctuations in intensity MUST come from molecular motion and must occur on a time scale slower than the sampling frequency.  Significant bleaching will lead to an overestimation of the molecular brightness.  Also note that non-fluctuating background signals will lead to an underestimation of molecular brightness.  For live cells, it is wise to measure a number of non-fluorescent (non-transformed) samples to find a background intensity level.  This can then be fixed in the analysis of fluorescent samples.  A sampling frequency of 20,000 Hz (corresponding to 50 us time bins) is typically fast enough for all measurements involving mEGFP in vivo.  This sampling frequency should be increased to 200,000 Hz to perform measurements on freely diffusing organic dyes (like fluorescein).

Another major consideration is the shape of the PSF.  Poorly aligned systems or systems with pinholes significantly greater than 1 airy unit will lead to distortions in the PSF and therefore a poor fit with PCH.  We have found that at typical molecular brightness levels for EGFP (less than 5,000 counter/sec/molecule), and pinhole diameters near 1 airy unit, these distortions do not significantly effect the PCH fit.  For much brighter species, the effects become greater due to the more thorough sampling of the out of focus regions of the PSF.  For two-photon systems, this is less of an issue because there is no pinhole.  For N&B analysis, distortions in the PSF will lead to changes in the gamma function.  These changes are likely to be small and can therefore be ignored.  Therefore N&B analysis provides a much more robust estimation of the brightness.

There are extra considerations for 2D PCH analysis.  It is extremely important that the objective in use be well color corrected.  Otherwise, the green and red foci will not overlap and the coincidence analysis will not work.  In addition, it is important to note that the red focus is larger than the green focus.  As a result, there will never be perfect coincidence of the two signals.  Finally, it is extremely important in dual color analysis to avoid bleaching or slow motions in the sample.  These motions are highly correlated between the two channels and therefore will appear as tightly bound species.  It is imperative to have a positive control in which the majority of the green and red species are bound to one another and a negative control in which none of the green and red species are bound.

Running the Program:

To install the program, simply extract all of the files from Jay Plugins.zip to your ImageJ plugins folder (c:\Program Files\ImageJ\plugins\ on windows systems).  If you do not have ImageJ, it is freely downloadable from http://rsbweb.nih.gov/ij/download.html.  The plugins “analysis pch” and “analysis pch 2D” will then appear under the ImageJ plugins menu.  When you click on this menu item, a dialog will appear asking for the sampling frequency and the psf type (3D Gaussian for confocal and Gaus-Lorentz^2 for two photon).  Then a file open dialog appears allowing for the selection of multiple confocor 3 raw files (hold down control and click multiple files).  There are a few other options for file import.  Short binary trajectory reads in a binary file with a list of unsigned 16-bit integers giving the number of photons per time bin.  PlotWindow trajectory works with trajectories generated by my simulation tools.  Ascii Text File reads a text file with a single column of integers giving the number of occurrences of 0,1,2,… photons or a tab delimited text file with an x axis.  Note that other than for confocor 3 raw files, the program does not care about the sampling frequency of the histogram.  The brightness is simply given in counts per time bin per molecule.  It is up to the user to make sure that the time bin is short enough to capture molecular fluctuations.  You can also select the option to show the trajectories for the files.  This of course only works with confocor 3 files and binary trajectories.  For the other options, there is no trajectory display.  At high sampling frequencies, it is difficult to see the trends in a trajectory, so you can bin the trajectory by a certain amount for display purposes.

Then the main program window appears along with several plot windows.  These plot windows display the average curve (the average of all selected curves), a selected curve with its fit, and the trajectory of the selected curve if chosen and available (only for 1D PCH).  The main window has check boxes along the left hand side to select the desired curves.  The non-selected curves will not be included in the average or the global fits.  Next the names of the files appear.  Then the N and B parameters appear and finally the chi squared values for the fits.  You can open and analyze up to 24 files at one time.  The bottom data set is always the average curve along with the average N and B parameters.  Clicking on the Fit Avg button brings up a parameter window for fitting.  If the check chi^2 box is selected, no fitting is performed, but the fit is updated to reflect the current parameters and the current chi squared is calculated.  In addition, if you click cancel, nothing is done.  This box is the only way to see the average fit parameters, so if you just want to see the results of the fit, open up the dialog and then click cancel to move on.  You can select the cells and copy into excel if you like.  Note that in order to add or remove species from the fit, simply unfix the number parameter for that species.  By default, one species is set up with the number and brightness parameters from the N&B analysis.  Once the fit is complete, the chi squared will appear beside the avg curve parameters on the main page.

The global fitting works similarly to the average fitting.  The major difference is that now one can vary, fix, or link parameters from a drop down menu in the dialog.  Linking parameters forces them to vary together.  If link is selected, the parameter is linked to the nearest leftward parameter that is not linked.  The starting value of that parameter will automatically be set to that of the linked parameter.  For example, if one wants to link all of the brightnesses together, the brightness for the first data set would be varied and that brightness for all other curves would be set to link.  If none of the parameters are set to link, global fitting simply performs fits on each of the data sets individually.  Note that with linking schemes the chi squared values for each curve are not explicitly defined.  The listed chi squareds are calculated from the parameters for each curve regardless of linking strategy.  Here, the global chi squared value represents the true chi squared for the fit.  Once again, in order to observe the parameters, one must open up the Global Fit dialog and then cancel to avoid doing another fit.  On the left hand side, there is an option to select the linking behavior for each row.  If Custom is selected, the linking parameters for the columns will take precedence.  If Link All is selected, all parameters but the leftmost will be linked (this will override Fixed parameters in all but the leftmost column).

One other option for linking is functional linking.  For now, this linking only occurs vertically in the table.  An example of this is fitting to a monomer and dimer species.  Since the dimer should be approximately twice as bright as the monomer, the brightness of the dimer species would be entered as “=2*e1g”.  Then FLink is selected from the linking menu.  In this case, the data will be fit to a model with two brightnesses, the second one twice as big as the other.  If any column is linked to a functionally linked column, it is also functionally linked vertically to its own e1g parameter (or whatever parameter is chosen).  This functional linking can also be used to fix the bleedthrough percentage in 2D PCH analysis.  Note that the formula parser for functional linking is quite simplistic.  There can be no parentheses.  As a result, if a sum of parameters is to be divided by a number or parameter, each parameter in the list must be divided individually.

The Reset Fit Params button simply resets all parameters to their estimated values.  These values are taken from the N and B analysis.  Note that if you remove a curve from the analysis, you need to press this button before the changes will be represented in the average fit dialog box.

The Undo Global Fit button simply undoes the results of the most recent global fit.

The Get Errors button performs support plane error analysis on any of the PCH (or 2D PCH) parameters.  Support plane error analysis is based on the F-test.  The F-test compares the chi squared value for two different fits to determine one model is significantly better than another.  F is defined as follows:

F=χ12χ22

The larger chi squared value is typically in the numerator giving F values greater than or equal to 1.  For a given set of numerator and denominator degrees of freedom and a specified confidence level, a limiting F value can be calculated.  If F is below this value, the denominator fit model is not a significant improvement over the numerator fit model.  If it is above the limiting F value the denominator is a significant improvement.  In order to use this theory to calculate errors, we can simply fix one parameter to values with increasing difference from its optimum fit value until this non-optimized chi squared value gives an F value equal to the limiting F value for the selected confidence interval.  This is done both above and below the parameter to give upper and lower confidence intervals.  Previous studies have shown that this tends to overestimate errors (Beechem).  As a result, the confidence interval is typically evaluated at the relatively low 67% confidence interval corresponding to approximately one standard deviation.

To run 2D analysis, select “analysis pch 2D” from the plugins menu.  Here the file import is a bit more complicated.  The program expects the file names for the two channels to be identical except for the last 5 digits.  The fifth digit from the end should contain a 1 or a 2 denoting which channel is being imported.  On confocor systems, the file names end with “Ch1.raw” or “Ch2.raw”.  For confocor systems, Ch2 is usually the green channel.  The initial dialog box has an option for “Ch2 is green” so that the colors are assigned correctly.  If an Ascii Text File is selected, the program expects a tab delimited text file representing the matrix of occurrences corresponding to each combination of channel intensities.  The program reads in the data as though column number corresponds to red intensity value and row number corresponds to green intensity value.  If “Ch2 is green” is selected, the columns will now represent green intensity and the rows will represent red intensity.

References:

Beechem, J. M. 1992. Global analysis of biochemical and biophysical data.  Meth. Enzymol. 210:37-54.

Chen, Y., J. D. Mueller, P. T. C. So, and E. Gratton. 1999. The photon counting histogram in fluorescence fluctuation spectroscopy. Biophys. J. 77:553-567.

Chen, Y., M. Tekmen, L. Hillesheim, J. Skinner, B. Wu, J. D. Mueller.  2005. Dual-Color Photon-Counting Histogram. Biophys. J. 88:2177-2192.

Digman, M. A., R. Dalal, A. R. Horwitz, and E. Gratton. 2008. Mapping the number of molecules and brightness in the laser scanning microscope. Biophys. J. 94:2320-2332.

Digman, M. A., P. W. Wiseman, C. Choi,  A. R. Horwitz, and E. Gratton. 2009. Stoichiometry of Molecular Complexes at Adhesions in Living Cells. Proc. Natl. Acad. Sci. U. S. A. 106:2170-2175.

Kask, P., K. Palo, D. Ullmann, and K. Gall. 1999. Fluorescence-intensity distribution analysis and its application in biomolecular detection technology. Proc. Natl. Acad. Sci. U. S. A. 96:13756-13761.

Kask, P. K. Palo, N. Fay, L. Brand, U. Mets, D. Ullmann, J. Jungmann, J. Pschorr, and K. Gall. 2000.  Two-Dimensional Fluorescence Intensity Distribution Analysis: Theory and Applications. Biophys. J. 78:1703-1713.

Mueller, J. D. 2004. Cumulant analysis in fluorescence fluctuation spectroscopy. Biophys. J. 86:3981-3992.

Qian, H., and E. L. Elson. 1990. Distribution of molecular aggregation by analysis of fluctuation moments. Proc. Natl. Acad. Sci. U. S. A. 87:5479-5483.

Thompson, N. L. 1991. Fluorescence Correlation Spectroscopy. In Top. Fluoresc. Spectrosc. J. R. Lakowicz, editor. Plenum Press, New York. 337-378.

Wu, B., Y. Chen, J. D. Mueller.  2006. Dual-Color Time-Integrated Fluorescence Cumulant Analysis. Biophys. J. 91:2687-2698.